## References

- Service Bus

https://azure.microsoft.com/zh-tw/services/service-bus/

- How to send cloud-to-device messages with IoT Hub and .Net

https://docs.microsoft.com/en-us/azure/iot-hub/iot-hub-csharp-csharp-c2d

- Service Bus Explorer

https://github.com/paolosalvatori/ServiceBusExplorer

## Requirements

- Finished the part 4 of HOL

- Two simulated wind turbines

- NuGet packages

  - Newtonsoft.Json for JSON in C#

  - WindowsAzure.ServiceBus for Service Bus

  - Microsoft.Azure.Devices (Service SDK for Azure IoT Devices)  for Cloud to Device message of IoT Hub

## Goals

- Create a Service Bus

  - Receive the alarm message from a Stream Analytics Job.

  - Send Cloud to Device command to the corresponding devices.

- Create an Azure Stream Analytics Job

  - Processing the telemetry data with device rules.

  - Trigger an alarm message to Service Bus when the alarm was detected.

## Step 1: Create a Service Bus for Alarm Message

- Search and Create the Service Bus

- Create a namespace

  - Name: iotworkshop112101 (for example)

  - Pricing tier: Basic

  - Use the existing Resource Group.

- Create a Queue

  - Name: cloud2device (must be fixed)

- Get the connection string of Service Bus

- Save the connection string of Service Bus for the later used.

## Step 2: Create new consumer groups of IoT Hub

- Find your IoT Hub

- Navigate to “Endpoints” of MESSAGEING, then click the “Events” to add a new consumer group.

- Add a new one for Alarm of Stream Analytics Job

  - Name: alarm (must be fixed)

- Add another consumer group for the telemetry push by Event Processor Host, and it will be used in the next HOL.

  - Name: telemetrypush (must be fixed)

  - Don’t forget to Save the changed properties.

## Step 3: Create a Stream Analytics Job for Alarm Message

- New a Stream Analytics Job

  - Job Name: alarmjob

  - Add an IoT Hub as the input

    - Input alias: iothub (must be fixed)

    - Source Type: Data stream

    - Source: IoT Hub

    - IoT Hub: select the used IoT Hub

    - Consumer group: alarm (not $Default)

    - The others should be set as below

  - It’s same as the HOL-4, also add a rule Blob as another one of inputs.

    - Input alias: devicerules (must be fixed)

    - Source Type: Reference data

    - Storage account: select the storage account as you provisioned

    - Container: devicerules (must be fixed)

    - Path pattern: {date}/{time}/devicerules.json (must be fixed)

    - Date format: YYYY-MM-DD (This format must be fixed)

    - Time format: HH-mm (This format must be fixed)

    - JSON and UTF-8 also should be fixed.

- Press the Create button.

  - Add a Service Bus as the output

    - Output alias: alarmsb (must be fixed)

    - Sink: Service bus Queue

    - Service bus namespace: select what you created

    - Queue name: cloud2device

  - Add a Query for processing the alarm message

    - Paste the following scripts to the query of Stream Analytics.

WITH StreamAvgData AS

(

SELECT

deviceId,

AVG(CAST(speed AS BIGINT)) AS AvgSpeed,

AVG(CAST(depreciation AS float)) AS AvgDepreciation,

System.TimeStamp AS LocalTime,

System.TimeStamp AS CreatedAt

FROM [iothub] TIMESTAMP BY time

GROUP BY

deviceId,

TumblingWindow(second, 30)

),

AlarmData AS

(

SELECT

Stream.deviceId AS IoTHubDeviceID,

'CutOutSpeed MessageID' AS [MessageID],

'CutOutSpeed' as [AlarmType],

Stream.AvgSpeed as [Reading],

Ref.[CutOutSpeed] as [Threshold],

Stream.LocalTime,

Stream.CreatedAt

FROM [StreamAvgData] Stream

JOIN [devicerules] Ref

ON

Stream.deviceId = Ref.[DeviceID]

WHERE Ref.[CutOutSpeed] IS NOT null AND Stream.AvgSpeed > Ref.[CutOutSpeed]

UNION ALL

SELECT

Stream2.deviceId AS IoTHubDeviceID,

'Repair MessageID' AS [MessageID],

'Repair' as [AlarmType],

Stream2.AvgDepreciation as [Reading],

Ref2.[Repair] as [Threshold],

Stream2.LocalTime,

Stream2.CreatedAt

FROM [StreamAvgData] Stream2

JOIN [devicerules] Ref2

ON

Stream2.deviceId = Ref2.[DeviceID]

WHERE Ref2.[Repair] IS NOT null AND Stream2.AvgDepreciation < Ref2.[Repair]

)

SELECT * INTO [alarmsb] FROM AlarmData

    - Save

- Start the Stream Analytics Job

  - Start now

  - Wait for the deployment

## Step 4: Run the Alert Service Bus Console App

- Build the Alert Service Bus Console App (05-HOL/Sample)

  - Unzip the AlertServiceBus.zip file and open the solution in VS

  - Update the connection string of App.config

    - ServiceBus.ConnectionString: the connection string of Service Bus

    - IoTHub.ConnectionString: the connection string of IoT Hub (for Cloud to Device command)

  - Press F5 to build and debug the console App

## Step 5: Run the simulated Linux and Windows wind turbines

- LinuxTurbine  - Run C Simple Sample of AMQP in SDK

cd ~/azure-iot-sdk-c/cmake/iotsdk_linux/serializer/samples/simplesample_amqp/

./simplesample_amqp

- WinTurbine - Run the executable file (SimulatedWindTurbine.exe) or debugging in the VS

  - The executable file should be located at

C:\Users\user\Documents\Visual Studio 2015\Projects\SimulatedWindTurbine\SimulatedWindTurbine\bin\Debug

## Step 7: Observe the results

- Check the console output of Service Bus. The alert should be shown when these conditions are triggered within 30 seconds:

  - Average wind speed > Cut-out Speed threshold (The default is 14)

  - Average depreciation rate < Repair threshold (The default is 0.5)

- Also, the device gets the Cloud-to-Device commands from IoT Hub when the alert is triggered.

  - The output of device

- The HOL 5 has been completed. We use another Azure Stream Analytics Job to do that to trigger on an alarm message when the device rule is detected in a real time. the alarm message will be sent to a queue of Service Bus for messaging queue. Then, we use a windows console app to receive these alarm messages from the queue and send C2D commands to the devices for the further used.

- The next hands-on lab, we are going to enjoy the power of data presentation in Power BI.

Let’s try to find out the insights from data.